Hacettepe University

Department of Electrical and Electronics Engineering

ELE432 - Advanced Digital Design

Preliminary Work 1

Murathan Yılmaz

2210357037

Lab Date: 25.03.2025

Q1)	Lab Duration

- Total Time Spent: 6 hours.

Q2)	State Transition Diagrams

The system is designed using two communicating Finite State Machines (FSMs):

A. Room FSM

This FSM tracks the player's location across seven distinct rooms.

- States: CAVE, TUNNEL, RIVER, STASH, DRAGON, VAULT, GRAVEYARD .

- Transitions: Movements are triggered by directional inputs (N, S, E, W).

- Logic: Access to the VAULT from the DRAGON's Den is only possible if the has_sword signal is high; otherwise, the state transitions to GRAVEYARD.

B. Sword FSM

This FSM monitors the possession of the Vorpal Sword.

- States: NO_SWORD (Initial), HAS_SWORD.

- Logic: Transitions to HAS_SWORD when the Room FSM signals sw_found (upon entering the STASH).

Q3)	Behavioral SystemVerilog Code

The implementation consists of a top-level module connecting the two sub-modules.

- adventure_game.sv: Connects the Room and Sword FSMs using internal signals sw_found and has_sword.

module adventure_game ( input logic clk, input logic reset, input logic n, s, e, w, output logic win, die ); logic sw_found; logic has_sword; // Room FSM room_fsm rooms ( .clk(clk), .reset(reset), .n(n), .s(s), .e(e), .w(w), .has_sword(has_sword), .sw_found(sw_found), .win(win), .die(die) ); // Sword FSM sword_fsm sword ( .clk(clk), .reset(reset), .sw_found(sw_found), .has_sword(has_sword) ); endmodule

- room_fsm.sv: Implements the room navigation logic using a 3-bit enumerated type and a synchronous always_ff block for state updates.

module room_fsm ( input logic clk, reset, input logic n, s, e, w, input logic has_sword, output logic sw_found, output logic win, die ); typedef enum logic [2:0] { CAVE, TUNNEL, RIVER, STASH, DRAGON, VAULT, GRAVEYARD } state_t; state_t current_state, next_state; // State Register always_ff @(posedge clk or posedge reset) begin if (reset) current_state <= CAVE; else current_state <= next_state; end // Next State Logic always_comb begin case (current_state) CAVE: next_state = (e) ? TUNNEL : CAVE; TUNNEL:next_state = (w) ? CAVE : (s) ? RIVER : TUNNEL; RIVER: next_state = (n) ? TUNNEL : (w) ? STASH : (e) ? DRAGON : RIVER; STASH: next_state = (e) ? RIVER : STASH; DRAGON:begin if (has_sword) next_state = VAULT; else next_state = GRAVEYARD; end VAULT: next_state = VAULT; // game is finished GRAVEYARD:next_state = GRAVEYARD; // game is finished default: next_state = CAVE; endcase end // Output Logic assign sw_found = (current_state == STASH); assign win = (current_state == VAULT); assign die = (current_state == GRAVEYARD); endmodule

- sword_fsm.sv: Manages the sword status, ensuring the sword remains possessed once found until a system reset occurs.

module sword_fsm ( input logic clk, input logic reset, input logic sw_found, output logic has_sword ); typedef enum logic { NO_SWORD, HAS_SWORD } state_t; state_t current_state, next_state; // State Register always_ff @(posedge clk or posedge reset) begin if (reset) current_state <= NO_SWORD; else current_state <= next_state; end // Next State Logic always_comb begin case (current_state) NO_SWORD: next_state = (sw_found) ? HAS_SWORD : NO_SWORD; HAS_SWORD: next_state = HAS_SWORD; default: next_state = NO_SWORD; endcase end // Output Logic assign has_sword = (current_state == HAS_SWORD); endmodule

Q4)	Testbench and Test Vectors

A self-checking testbench was developed to verify the design.

- Scenario 1 (Die): The player moves directly from CAVE -> TUNNEL -> RIVER -> DRAGON without the sword, resulting in the die output being asserted.

- Scenario 2 (Win): After a reset, the player moves CAVE -> TUNNEL -> RIVER -> STASH (obtaining the sword) -> RIVER -> DRAGON -> VAULT, asserting the win output.

`timescale 1ns/1ps module tb_adventure_game(); logic clk, reset, n, s, e, w; logic win, die; adventure_game dut (.*); // Generate clock signal initial clk = 0; always #5 clk = ~clk; // Helper for clearing signal task clear_inputs(); {n, s, e, w} = 4'b0000; endtask initial begin // --- scenario 1 reset = 1; clear_inputs(); #20 reset = 0; @(posedge clk); e = 1; #10; clear_inputs(); // Twisty Tunnel @(posedge clk); s = 1; #10; clear_inputs(); // Rapid River @(posedge clk); e = 1; #10; clear_inputs(); // Dragon's Den #10; // --- scenario 2 #20 reset = 1; #20 reset = 0; // restart @(posedge clk); e = 1; #10; clear_inputs(); // Tunnel @(posedge clk); s = 1; #10; clear_inputs(); // River @(posedge clk); w = 1; #10; clear_inputs(); // Stash (sword is taken) @(posedge clk); e = 1; #10; clear_inputs(); // Back to River @(posedge clk); e = 1; #10; clear_inputs(); // Dragon's Den @(posedge clk); e = 1; #10; clear_inputs(); // Victory Vault #10; #50 $stop; end endmodule

Q5)	Simulation Waveforms

The simulation was performed using QuestaSim.

- Verification: The waveforms display the clk, reset, directional inputs, and the current_state signal.

- Result: All test vectors passed the self-checking mechanism, confirming that the FSM transitions correctly match the game logic and map requirements.

Q6)	RTL Viewer Schematics

The synthesized schematic confirms the structural integrity of the design.

- Analysis: The RTL Viewer displays the two FSM blocks and their bidirectional communication.

- Expectation: The registers (flip-flops) and combinational logic (multiplexers) correctly represent the intended behavioral idiom.